SWE4303 Computing Infrastructure

Week 5 | Session 2 Lab Handout

Protocol Analysis Lab: Observing Application Exchanges and Security Differences

Cisco acknowledgement: This lab is adapted for teaching in SWE4303 using Cisco Packet Tracer and selected Cisco Networking Academy lab approaches. Cisco is acknowledged as the original source of the underlying platform and lab style used here for educational delivery.

Assessment preparation note: This lab uses a fixed teaching network so that everyone can practise the same skills. Assessment 1 and Assessment 2 use personalised or scenario-based evidence, so do not copy these lab addresses into your assessment unless instructed.

Purpose of this lab

This lab helps you move from Week 5 lecture ideas into practical protocol observation. You will build a small routed network in Cisco Packet Tracer, configure and test selected services, observe protocol-related behaviour, and explain what the evidence proves.

The purpose is not to become a protocol analyst or a security specialist. The purpose is to understand how common application and management protocols support networked services, and how to interpret protocol behaviour using evidence.

What you should be able to do by the end

- build a simple two-subnet topology in Packet Tracer

- verify reachability before testing application services

- configure and test a basic HTTP web service

- observe HTTP and TCP events in Packet Tracer Simulation mode

- compare HTTP and HTTPS carefully without over-claiming what HTTPS proves

- configure SSH for protected remote administration and compare it with Telnet behaviour

- test FTP service behaviour and explain why protocol suitability matters

- configure email users and test SMTP sending with POP3 mailbox access

- record evidence and explain what each protocol test proves and does not prove

Software and resources

- Cisco Packet Tracer installed and working

- one PC or laptop with enough screen space to view the workspace and device windows comfortably

- this handout open beside Packet Tracer if possible

- time to save work carefully after each major stage

Before you start

- Open Packet Tracer and make sure you are in Realtime mode.

- Save your file early with a clear name such as Week5_Session2_YourName.pkt.

Save again after each major part. Do not use a pre-built activity file. You are expected to build the topology yourself.

Use Simulation mode only when instructed. Most configuration should be completed in Realtime mode first.

Cable guidance for this lab

| Connection | Cable to use | Notes |
| --- | --- | --- |
| PC to switch | Copper Straight-Through | Use this for all PC-to-switch links. |
| Server to switch | Copper Straight-Through | Use this for the server-to-switch link. |
| Switch to router | Copper Straight-Through | Use this for switch-to-router links. |
| PC to router console | Light blue Console cable | Optional only if your tutor asks you to use console access. |

Part 0 — Understand the protocol-observation brief

A small training centre uses several internal services. Staff access a web portal, transfer files to a server, manage a network device remotely, and use internal email. The organisation wants these services to be testable and wants technical staff to explain what each protocol exchange proves.

In Week 4, you tested whether users could find and reach services. In this lab, you will add a new question: once the service is reachable, what protocol behaviour is taking place?

Key idea: a protocol test should produce evidence. Strong evidence explains the protocol purpose, the client/server roles, the security property involved, and the limits of what the test proves.

Services you will observe

| Service area | Protocol(s) | Main practical focus |
| --- | --- | --- |
| Web access | HTTP / HTTPS | Request-response behaviour and protection in transit. |
| Remote administration | Telnet / SSH | Why protected management access matters. |
| File transfer | FTP / SFTP | File-transfer behaviour and suitability for sensitive data. |
| Email | SMTP / POP3, with IMAP as conceptual comparison | Sending mail versus accessing mailbox messages. |
| TCP/IP model | Application, Transport, Internet, Network Access | How application protocols depend on lower-layer delivery. |

Packet Tracer supports SMTP and POP3 for the email activity. It does not support IMAP in this lab environment. IMAP is therefore discussed conceptually as a mailbox access and synchronisation protocol, not configured as a practical task.

Subnet planning table

The original 192.168.50.0/24 teaching network is divided into four equal /26 subnet blocks. In this lab, the first two subnet blocks are used and the remaining two are kept for future growth.

| Subnet purpose | Network address | Prefix | Subnet mask | Gateway | Use |
| --- | --- | --- | --- | --- | --- |
| Administration LAN | 192.168.50.0 | /26 | 255.255.255.192 | 192.168.50.1 | Admin-PC and management access |
| Services LAN | 192.168.50.64 | /26 | 255.255.255.192 | 192.168.50.65 | Services-Server and User-PC |
| Future LAN 1 | 192.168.50.128 | /26 | 255.255.255.192 | Not used | Reserved |
| Future LAN 2 | 192.168.50.192 | /26 | 255.255.255.192 | Not used | Reserved |

Device addressing table

| Device | Interface | IPv4 address | Subnet mask | Default gateway | Notes |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.50.1 | 255.255.255.192 | N/A | Administration LAN gateway |
| R1 | G0/1 | 192.168.50.65 | 255.255.255.192 | N/A | Services LAN gateway |
| S1 | VLAN 1 | 192.168.50.2 | 255.255.255.192 | 192.168.50.1 | Administration switch management |
| S2 | VLAN 1 | 192.168.50.66 | 255.255.255.192 | 192.168.50.65 | Services switch management |
| Admin-PC | NIC | 192.168.50.10 | 255.255.255.192 | 192.168.50.1 | Administration client |
| User-PC | NIC | 192.168.50.74 | 255.255.255.192 | 192.168.50.65 | Services-side user client |
| Services-Server | NIC | 192.168.50.70 | 255.255.255.192 | 192.168.50.65 | HTTP, FTP and Email services |

Check your understanding

Why should reachability be tested before HTTP, FTP, SSH, or email services are blamed?

Answer: _______________________________________________________________________

Why is it useful to keep the topology simple in a protocol-observation lab?

Answer: _______________________________________________________________________

Why is IMAP discussed but not configured in this Packet Tracer lab?

Answer: _______________________________________________________________________

Part 1 — Build the topology

Devices to place

Place the following devices in Packet Tracer:

- 1 router — name it R1 (use a 2911 router if available)

- 2 switches — name them S1 and S2 (use 2960 switches)

- 1 server — name it Services-Server

- 2 PCs — name them Admin-PC and User-PC

Use a router with at least two Gigabit Ethernet interfaces. If your router uses different interface names, adjust the commands carefully. For example, some routers use G0/0 and G0/1; others use G0/0/0 and G0/0/1.

Suggested layout

- Administration devices on the left

- Services devices on the right

- Router in the centre

- Switches between the router and end devices

- Services-Server connected to the Services LAN switch

Figure 1. Protocol-observation topology to build in Packet Tracer.

Connect the devices

Use Copper Straight-Through cables.

| Link | Suggested interface |
| --- | --- |
| Admin-PC to S1 | PC FastEthernet0 to S1 FastEthernet0/6 |
| S1 to R1 | S1 GigabitEthernet0/1 to R1 GigabitEthernet0/0 |
| S2 to R1 | S2 GigabitEthernet0/1 to R1 GigabitEthernet0/1 |
| User-PC to S2 | PC FastEthernet0 to S2 FastEthernet0/6 |
| Services-Server to S2 | Server FastEthernet0 to S2 FastEthernet0/18 |

Save your Packet Tracer file before continuing.

Part 2 — Configure base addressing

This lab uses static addressing so that you can focus on protocol behaviour. DHCP was covered in Week 4 and should not become the main purpose of this lab.

Step 1 — Configure the PCs and server

For each PC and the server, open Desktop > IP Configuration and enter the IPv4 address, subnet mask, and default gateway from the addressing table.

Check carefully that Admin-PC uses gateway 192.168.50.1, while User-PC and Services-Server use gateway 192.168.50.65.

Step 2 — Configure router R1

In this step, you are configuring R1 so that it can act as the default gateway for both subnet areas.

Router> enable Router# configure terminal Router(config)# hostname R1 R1(config)# no ip domain-lookup R1(config)# interface gigabitethernet0/0 R1(config-if)# description Administration LAN gateway R1(config-if)# ip address 192.168.50.1 255.255.255.192 R1(config-if)# no shutdown R1(config-if)# exit R1(config)# interface gigabitethernet0/1 R1(config-if)# description Services LAN gateway R1(config-if)# ip address 192.168.50.65 255.255.255.192 R1(config-if)# no shutdown R1(config-if)# end R1# copy running-config startup-config

If Packet Tracer asks for the destination filename, press Enter.

What the router configuration is doing

| Command or command group | Why it is used |
| --- | --- |
| hostname R1 | Gives the router a clear name so outputs and prompts are easier to read. |
| no ip domain-lookup | Prevents Packet Tracer from pausing while trying to treat mistyped commands as domain names. |
| interface gigabitethernet0/0 | Selects the router interface connected to the Administration LAN. |
| ip address 192.168.50.1 255.255.255.192 | Assigns the gateway address for the Administration subnet. |
| interface gigabitethernet0/1 | Selects the router interface connected to the Services LAN. |
| ip address 192.168.50.65 255.255.255.192 | Assigns the gateway address for the Services subnet. |
| no shutdown | Turns a router interface on. Without this, the interface may remain administratively down. |
| copy running-config startup-config | Saves the configuration so it is not lost if the device restarts. |

Step 3 — Configure switch management addresses

The switches forward frames inside each LAN; they do not route traffic between subnets in this lab. However, giving them management IP addresses lets you test management reachability and prepares for SSH practice.

Switch> enable Switch# configure terminal Switch(config)# hostname S1 S1(config)# no ip domain-lookup S1(config)# interface vlan 1 S1(config-if)# ip address 192.168.50.2 255.255.255.192 S1(config-if)# no shutdown S1(config-if)# exit S1(config)# ip default-gateway 192.168.50.1 S1(config)# end S1# copy running-config startup-config

Switch> enable Switch# configure terminal Switch(config)# hostname S2 S2(config)# no ip domain-lookup S2(config)# interface vlan 1 S2(config-if)# ip address 192.168.50.66 255.255.255.192 S2(config-if)# no shutdown S2(config-if)# exit S2(config)# ip default-gateway 192.168.50.65 S2(config)# end S2# copy running-config startup-config

Save your Packet Tracer file.

Check your understanding

Why do the two router interfaces need addresses from different subnets?

Answer: _______________________________________________________________________

Why is the switch default gateway needed for management traffic outside its own subnet?

Answer: _______________________________________________________________________

Why are we using static addresses in this lab rather than DHCP?

Answer: _______________________________________________________________________

Part 3 — Verify basic reachability

Do not jump straight to application services. Test in order. This is the same evidence-based habit used in earlier weeks.

Why the tests are staged

| Stage | What you are checking | Why it matters |
| --- | --- | --- |
| Check IP configuration | Does each device have the correct IP, mask and gateway? | Later tests depend on usable host configuration. |
| Ping local gateway | Can each host reach its local router interface? | Confirms the host has a working path out of its subnet. |
| Ping server by IP | Can clients reach 192.168.50.70? | Confirms server reachability without relying on an application protocol. |

Test 1 — Check IP configuration

On Admin-PC and User-PC, open Desktop > Command Prompt and run:

ipconfig

| Client | IP address correct? | Gateway correct? | Evidence or note |
| --- | --- | --- | --- |
| Admin-PC |  |  |  |
| User-PC |  |  |  |

Test 2 — Ping the local default gateway

On Admin-PC, run:

ping 192.168.50.1

On User-PC, run:

ping 192.168.50.65

| Test | Successful? | Evidence or note |
| --- | --- | --- |
| Admin-PC to 192.168.50.1 |  |  |
| User-PC to 192.168.50.65 |  |  |

Test 3 — Ping the Services-Server by IP address

From Admin-PC and User-PC, run:

ping 192.168.50.70

| Test | Successful? | Evidence or note | What this proves |
| --- | --- | --- | --- |
| Admin-PC to Services-Server |  |  |  |
| User-PC to Services-Server |  |  |  |

Part 4 — Configure and test HTTP web access

In this part, you will configure a simple HTTP web service on Services-Server and test whether the client can access it.

Step 1 — Configure HTTP on Services-Server

- Click Services-Server.

- Open the Services tab.

- Select HTTP.

- Make sure HTTP is On.

If your Packet Tracer version allows editing the index page, change the page text to Training Centre Services Portal.

Step 2 — Test HTTP by IP address

- Click Admin-PC > Desktop > Web Browser.

- In the URL field, enter http://192.168.50.70 and click Go.

- Repeat the same test from User-PC if time allows.

| Client | URL tested | Successful? | Evidence or note | What this proves |
| --- | --- | --- | --- | --- |
| Admin-PC | http://192.168.50.70 |  |  |  |
| User-PC | http://192.168.50.70 |  |  |  |

Check your understanding

What does a successful HTTP browser test prove?

Answer: _______________________________________________________________________

What does it not prove?

Answer: _______________________________________________________________________

Which device is the client, and which device is the server in this test?

Answer: _______________________________________________________________________

Part 5 — Observe HTTP and TCP behaviour in Simulation mode

In this part, you will use Packet Tracer Simulation mode to observe that HTTP is an application protocol supported by TCP. This is an observation task, not a deep packet-forensic task.

Step 1 — Prepare the simulation

- Switch to Simulation mode.

- Open the Simulation Panel.

- Select Edit Filters.

- Turn off unnecessary filters and select only TCP and HTTP.

- On Admin-PC, open the Web Browser and request http://192.168.50.70 again.

Step 2 — Observe the event list

- Use Capture/Forward to move the events step by step.

- Watch for TCP events before the HTTP event appears.

- Open one HTTP or TCP PDU and look for source, destination, and protocol details.

Do not try to memorise every field. Focus on what the event proves.

| Observation | Record what you saw |
| --- | --- |
| Which protocol events appeared before HTTP? |  |
| Which device sent the request? |  |
| Which device responded? |  |
| What evidence shows this is a web service exchange? |  |

Interpretation

A successful HTTP event helps prove that the web service can respond. It does not prove that communication is protected, that the application is well designed, or that the user is authorised to access every page.

Part 6 — Compare HTTP and HTTPS carefully

HTTPS keeps the same broad web purpose as HTTP, but adds protection in transit. Packet Tracer behaviour may vary by version, so this part is written as a careful observation and interpretation task.

Step 1 — Check HTTPS availability

- On Services-Server, open Services > HTTP.

- If HTTPS can be enabled in your Packet Tracer version, turn it On.

- From Admin-PC, test http://192.168.50.70.

- From Admin-PC, test https://192.168.50.70.

Record what happens. If HTTPS is not available or does not behave clearly in your version, record that limitation rather than guessing.

| Test | Expected result | Actual result | Evidence or note |
| --- | --- | --- | --- |
| HTTP access | Web page loads if HTTP service is on |  |  |
| HTTPS access | Protected web access works only if HTTPS is supported and enabled |  |  |

Check your understanding

Does HTTPS change the broad purpose of the web service?

Answer: _______________________________________________________________________

What protection does HTTPS add?

Answer: _______________________________________________________________________

Why should a login or protected page normally use HTTPS rather than HTTP?

Answer: _______________________________________________________________________

Does HTTPS prove that the whole web application is secure?

Answer: _______________________________________________________________________

Part 7 — Compare Telnet and SSH for remote administration

In this part, you will first configure basic Telnet access to demonstrate that unprotected remote administration can technically work. You will then replace Telnet with SSH-only access to show the more appropriate protected management method. The purpose is to understand that remote access should be protected, especially where credentials and configuration commands are involved.

Step 1 — Demonstrate basic Telnet access first

Click R1 > CLI and enter the following commands. These commands deliberately enable basic Telnet access so that you can observe the difference between technical access and protected administration.

R1# configure terminal R1(config)# enable secret class R1(config)# line vty 0 4 R1(config-line)# password class R1(config-line)# login R1(config-line)# transport input telnet R1(config-line)# end R1# copy running-config startup-config

What this Telnet configuration is doing

| Command or setting | Why it matters |
| --- | --- |
| enable secret class | Sets the privileged EXEC password so a remote administrator can enter enable mode after login. This is separate from the VTY login password. |
| line vty 0 4 | Selects the virtual terminal lines used for remote CLI access such as Telnet and SSH. |
| password class | Sets the VTY line password used for this basic Telnet demonstration. |
| login | Requires users to enter the VTY password before accessing the router CLI. |
| transport input telnet | Allows Telnet access on the VTY lines. This is deliberately used here only as an insecure comparison point. |
| copy running-config startup-config | Saves the configuration so it is not lost if the device restarts. |

Step 2 — Test Telnet from Admin-PC

On Admin-PC, open Desktop > Command Prompt and enter:

telnet 192.168.50.1

When prompted for the Telnet password, use:

class

You should reach the router user prompt:

R1>

To enter privileged mode, type:

enable

When prompted for the enable password, use:

class

You should then reach the privileged router prompt:

R1#

| Test | Expected result | Actual result | Evidence or note |
| --- | --- | --- | --- |
| Initial Telnet to R1 | Telnet login succeeds after VTY access is configured |  |  |
| Enable mode after Telnet login | R1# prompt is reached using the enable secret |  |  |

Interpretation

This test shows that Telnet can technically provide remote CLI access once VTY access is configured. It does not show that the access method is suitable for protected administration. Telnet is used here only as a contrast before configuring SSH.

Step 3 — Configure SSH on R1

Now replace Telnet with SSH-only access. Click R1 > CLI and enter the following commands. If your Packet Tracer version asks for a key modulus after the crypto key command, enter 1024.

R1# configure terminal R1(config)# enable secret class R1(config)# ip domain-name training.com R1(config)# username admin secret AdminPass123! R1(config)# crypto key generate rsa 1024 R1(config)# ip ssh version 2 R1(config)# line vty 0 4 R1(config-line)# login local R1(config-line)# transport input ssh R1(config-line)# exec-timeout 6 0 R1(config-line)# end R1# copy running-config startup-config

What this SSH configuration is doing

| Command or setting | Why it matters |
| --- | --- |
| enable secret class | Sets the privileged EXEC password so a remote administrator can enter enable mode after login. |
| ip domain-name training.com | Provides the domain name needed when generating RSA keys. |
| username admin secret AdminPass123! | Creates a local administrator account with an encrypted secret. |
| crypto key generate rsa | Generates keys needed for SSH operation. |
| ip ssh version 2 | Uses SSH version 2 where supported. |
| login local | Tells the VTY lines to use the local username database. |
| transport input ssh | Allows SSH access and rejects Telnet access on the VTY lines. |
| exec-timeout 6 0 | Closes idle remote sessions after 6 minutes. |

Step 4 — Test Telnet and SSH from Admin-PC

On Admin-PC, open Desktop > Command Prompt. After SSH-only access has been configured, first try Telnet again:

telnet 192.168.50.1

The Telnet attempt should now fail or be rejected because R1 is configured to accept SSH only. Now try SSH:

ssh -l admin 192.168.50.1

When prompted, use the password:

AdminPass123!

After SSH login, you should reach R1>. To enter privileged mode, type:

enable

When prompted for the enable password, use:

class

| Test | Expected result | Actual result | Evidence or note |
| --- | --- | --- | --- |
| Telnet to R1 after SSH-only configuration | Connection fails or is rejected |  |  |
| SSH to R1 | SSH login succeeds with correct username and password |  |  |
| Enable mode after SSH login | R1# prompt is reached using the enable secret |  |  |

Check your understanding

What does successful SSH login prove?

Answer: _______________________________________________________________________

Why is Telnet not suitable for protected remote administration?

Answer: _______________________________________________________________________

Why does remote administration need stronger protection than ordinary web browsing?

Answer: _______________________________________________________________________

Part 8 — Configure and test FTP service behaviour

In this part, you will configure FTP on Services-Server and test a file-transfer session. The point is not only whether the service works. The point is also whether the protocol is suitable for the type of data being transferred.

Step 1 — Configure FTP on Services-Server

- Click Services-Server.

- Open Services > FTP.

- Turn the FTP service On.

- Create a user account with username student and password class.

Give the user permission to read, write, delete, rename and list files if Packet Tracer asks for permissions.

Step 2 — Connect to FTP from Admin-PC

On Admin-PC, open Desktop > Command Prompt and enter:

ftp 192.168.50.70

When prompted, use:

Username: student Password: class

At the ftp> prompt, enter:

dir

If your Packet Tracer version supports the available file workflow, you may also try get or put commands. The core evidence for this lab is the FTP connection, authentication and directory listing.

| FTP action | Expected result | Actual result | Evidence or note |
| --- | --- | --- | --- |
| Connect to FTP server | FTP session opens |  |  |
| Login as student | Login succeeds |  |  |
| Run dir | Directory listing appears |  |  |
| Optional get/put | File transfer succeeds if supported |  |  |

Interpretation

FTP can support file transfer, but it is not suitable where credentials or sensitive data need protection. SFTP is the stronger protocol choice when file transfer needs to be protected. Packet Tracer may not provide a full SFTP service workflow in this lab, so SFTP is treated mainly as the appropriate secure comparison.

Check your understanding

What does a successful FTP login prove?

Answer: _______________________________________________________________________

Why does successful file transfer not automatically mean the protocol choice is suitable?

Answer: _______________________________________________________________________

When would SFTP be a better protocol choice than FTP?

Answer: _______________________________________________________________________

Part 9 — Configure and test email sending and mailbox access

In this part, you will configure a simple Packet Tracer email service. The purpose is to observe that email is not one protocol. Sending and mailbox access involve different protocol roles.

Step 1 — Configure email service on Services-Server

- Click Services-Server.

- Open Services > EMAIL.

- Turn SMTP and POP3 On if they are not already enabled.

- Set the domain name to training.com.

- Create user admin with password class.

- Create user user with password class.

Step 2 — Configure email on Admin-PC

On Admin-PC, open Desktop > Email and configure the account as follows.

| Setting | Value |
| --- | --- |
| Your Name | Admin User |
| Email Address | admin@training.com |
| Incoming Mail Server | 192.168.50.70 |
| Outgoing Mail Server | 192.168.50.70 |
| User Name | admin |
| Password | class |

Step 3 — Configure email on User-PC

On User-PC, open Desktop > Email and configure the account as follows.

| Setting | Value |
| --- | --- |
| Your Name | User One |
| Email Address | user@training.com |
| Incoming Mail Server | 192.168.50.70 |
| Outgoing Mail Server | 192.168.50.70 |
| User Name | user |
| Password | class |

Step 4 — Send and retrieve a test message

- From Admin-PC, compose a message to user@training.com.

- Use a clear subject such as Week 5 protocol test.

- Send the message.

- On User-PC, use the Email tool to receive or check mail.

Record whether the message was received.

| Action | Expected result | Actual result | Evidence or note |
| --- | --- | --- | --- |
| Admin-PC sends email | Message sends successfully |  |  |
| User-PC retrieves email | Message appears in mailbox |  |  |
| Interpret protocol role | SMTP supports sending; POP3 supports mailbox retrieval/access |  |  |

Step 5 — Observe email traffic in Simulation mode

If time allows, switch to Simulation mode and filter for SMTP, POP3 and TCP. Send or retrieve the message again and observe the event list.

| Observation | Record what you saw |
| --- | --- |
| Which protocol appeared when sending email? |  |
| Which protocol appeared when retrieving mailbox messages? |  |
| What transport protocol supported the email traffic? |  |
| What does this prove about email being more than one protocol? |  |

Important note about IMAP

IMAP is not configured in this lab because Packet Tracer does not support IMAP in this activity environment. You should still understand the concept: IMAP supports mailbox access and synchronisation across devices, while POP is commonly used in a more download-oriented retrieval model.

Check your understanding

Which protocol is mainly used for sending email?

Answer: _______________________________________________________________________

Which Packet Tracer protocol is used here for mailbox retrieval/access?

Answer: _______________________________________________________________________

Why does successful sending not prove that mailbox access is working?

Answer: _______________________________________________________________________

Why might IMAP be more suitable where users access the same mailbox from several devices?

Answer: _______________________________________________________________________

Part 10 — Complete the evidence and interpretation table

Complete the table below. Do not only record whether each test worked. Explain what the evidence proves and what it does not prove.

| Test | Expected result | Actual result | Evidence collected | What this proves | What this does not prove |
| --- | --- | --- | --- | --- | --- |
| Ping Services-Server | Successful reply |  |  | Path exists to the server | Application service works |
| HTTP browser access | Web page loads |  |  | HTTP service responds | Communication is protected |
| HTTPS test | Works only if supported/enabled |  |  | Protected web option may be available | Whole application is secure |
| Initial Telnet access | Remote CLI opens after VTY Telnet access is configured |  |  | Telnet can technically provide remote CLI access | Telnet is suitable for protected administration |
| SSH login | Remote session opens |  |  | SSH is enabled and credentials work | All security controls are complete |
| Telnet after SSH-only configuration | Fails or is rejected |  |  | Telnet is not accepted for management after SSH-only configuration | Initial Telnet access was possible only because VTY Telnet access was deliberately configured earlier |
| FTP login/listing | FTP session opens and lists files |  |  | FTP service is available | FTP is suitable for sensitive data |
| Email send/retrieve | Message can be sent and retrieved |  |  | SMTP and POP3 roles can be observed | IMAP synchronisation is proven |

Part 11 — Controlled interpretation cases

Use your lab evidence and the Week 5 lecture logic to answer the following cases.

Case 1 — HTTP page works, protected page fails

A client can open http://example.com. The same client cannot open http://example.com/login.

Most likely: __________________________________________________________________

Reason: _______________________________________________________________________

What to test next: _____________________________________________________________

Case 2 — Remote access works, but not securely

A technician can connect to a network device using Telnet. The same device does not accept SSH.

Most likely: __________________________________________________________________

Reason: _______________________________________________________________________

What to test next: _____________________________________________________________

Case 3 — Email sends, but mailbox access fails

A user can send an email from a mail client. The same user cannot view received messages in the mailbox.

Most likely: __________________________________________________________________

Reason: _______________________________________________________________________

What to test next: _____________________________________________________________

Evidence checklist

Before you finish, make sure you have the following evidence. Your tutor may ask to see the Packet Tracer file, screenshots, command outputs, or completed tables.

| Evidence item | Completed? |
| --- | --- |
| Packet Tracer file saved with a clear Week 5 filename | [ ] |
| Addressing and reachability evidence completed | [ ] |
| HTTP browser test completed and interpreted | [ ] |
| HTTP/TCP behaviour observed in Simulation mode | [ ] |
| HTTPS availability or limitation recorded carefully | [ ] |
| Initial Telnet, SSH-only configuration and SSH login tested and explained | [ ] |
| FTP service evidence recorded | [ ] |
| Email SMTP/POP3 evidence recorded | [ ] |
| Final evidence table and interpretation cases completed | [ ] |

If you get stuck

Check in this order:

- Are the cables connected to the correct devices and interfaces?

- Are the router interfaces turned on with no shutdown?

- Is each device using the correct IP address, subnet mask and default gateway?

- Can each PC ping its own default gateway?

- Can each PC ping Services-Server by IP address?

- Is the server using the correct static IP address and default gateway?

- Is the HTTP service turned on before you test the browser?

- Are the Simulation mode filters set only to the protocol you are trying to observe?

- For SSH, has R1 been given an enable secret, domain name, local username, RSA keys and VTY lines set to login local and transport input ssh?

- Are you using the correct SSH syntax: ssh -l admin 192.168.50.1?

- For FTP, is the FTP service turned on and is the student user account configured with the correct password?

- For email, are SMTP and POP3 turned on, is the domain set to training.com, and are the client incoming/outgoing server fields set to 192.168.50.70?

- If a service does not appear to update, close and reopen the client tool or return briefly to Realtime mode before testing again.

Do not change many settings at the same time. Change one thing, test it, and then continue.

Key idea from this lab

A protocol test is useful only when it is interpreted carefully. HTTP, SSH, FTP and email traffic can show that a service is available, but each test has limits. Strong infrastructure reasoning explains the protocol purpose, the client/server roles, the security property involved, and what the evidence does or does not prove.

Consolidate and look ahead

After the lab, complete the Week 5 quiz on matching protocols to purpose and security characteristics.

Self-study: write a brief reflection on why protocol choice matters in enterprise infrastructure.

Keep your Packet Tracer file because the protocol-observation and evidence logic may help you when explaining protocols and network services in Assessment 1. It will also support later thinking about client/server implementation evidence in Assessment 2.

End of document